# GUARDX Health Score Example – 65/100 Predictive Maintenance Scenario

This document presents a detailed example of the GUARDX ESP Health Score calculation resulting in a final health score near 62.9/100. The example demonstrates how GUARDX evaluates multiple simultaneous degradation mechanisms across surface equipment, power quality, cable system, motor, seal section, pump, intake system, and fluid/reservoir behavior.

The purpose of this example is to illustrate how predictive maintenance logic combines severity, AI confidence, persistence, and domain weighting to generate a realistic operational health index for the complete ESP system.

## 1. Core Health Score Equation

S_health = 100 - D_total

Where:

D_total = Σ(W_d × R_d)

Where:
- S_health = Final ESP Health Score
- D_total = Total weighted degradation
- W_d = Domain weighting factor
- R_d = Risk score of each domain

## 2. Domain Risk Equation

R_d = Σ(S_i × C_i × P_i × E_i)

Where:
- S_i = Severity of the fault
- C_i = AI confidence level
- P_i = Persistence factor
- E_i = Exclusion logic factor

## 3. Meaning of the Calculation Elements

Severity:
Represents how serious the abnormality is right now. Severity is derived from the magnitude of deviation from the healthy operating baseline.

Confidence:
Represents how certain the AI engine is regarding the diagnosis. Confidence is generated using multi-sensor correlation, historical fingerprint matching, and differential diagnosis logic.

Persistence:
Represents how long the abnormal condition has continued. Persistence prevents false alarms caused by temporary startup transients or short disturbances.

Exclusion Logic:
Used to eliminate impossible diagnoses. For example, if intake pressure fluctuates continuously, the pump is still rotating, therefore a broken shaft diagnosis may be excluded.

## 4. Domain Weighting

| Domain | Weight |
| --- | --- |
| Surface Equipment / VSD | 10% |
| Power Quality | 10% |
| Cable / Surface Connection | 10% |
| Motor | 15% |
| Seal Section | 15% |
| Pump | 20% |
| Intake / Gas Handling | 10% |
| Fluid / Reservoir | 10% |

## 5. Full Multi-Domain Example Calculation

| Domain | Problem | Severity | Confidence | Persistence | Risk Calculation | Risk |
| --- | --- | --- | --- | --- | --- | --- |
| Surface | Mild VSD thermal stress | 45 | 0.75 | 0.50 | 45×0.75×0.50 | 16.9 |
| Surface | Minor inverter ringing | 35 | 0.70 | 0.40 | 35×0.70×0.40 | 9.8 |
| Surface Total |  |  |  |  | 26.7×10% | 2.7 |
| Power | Moderate harmonic heating | 50 | 0.80 | 0.60 | 50×0.80×0.60 | 24.0 |
| Power Total |  |  |  |  | 24.0×10% | 2.4 |
| Cable | Early cable insulation aging | 40 | 0.65 | 0.50 | 40×0.65×0.50 | 13.0 |
| Cable Total |  |  |  |  | 13.0×10% | 1.3 |
| Motor | Current imbalance | 50 | 0.85 | 0.70 | 50×0.85×0.70 | 29.8 |
| Motor | Mild rotor asymmetry | 40 | 0.70 | 0.60 | 40×0.70×0.60 | 16.8 |
| Motor Total |  |  |  |  | 46.6×15% | 7.0 |
| Seal | Early thrust-bearing wear | 55 | 0.80 | 0.70 | 55×0.80×0.70 | 30.8 |
| Seal | Minor oil contamination | 35 | 0.60 | 0.50 | 35×0.60×0.50 | 10.5 |
| Seal Total |  |  |  |  | 41.3×15% | 6.2 |
| Pump | Moderate scaling | 60 | 0.85 | 0.75 | 60×0.85×0.75 | 38.3 |
| Pump | Efficiency degradation | 50 | 0.90 | 0.70 | 50×0.90×0.70 | 31.5 |
| Pump Total |  |  |  |  | 69.8×20% | 14.0 |
| Intake | Intermittent gas lock | 45 | 0.85 | 0.50 | 45×0.85×0.50 | 19.1 |
| Intake Total |  |  |  |  | 19.1×10% | 1.9 |
| Fluid | Early asphaltene deposition | 40 | 0.65 | 0.60 | 40×0.65×0.60 | 15.6 |
| Fluid Total |  |  |  |  | 15.6×10% | 1.6 |

## 6. Final Weighted Contributions

| Domain | Weighted Contribution |
| --- | --- |
| Surface | 2.7 |
| Power | 2.4 |
| Cable | 1.3 |
| Motor | 7.0 |
| Seal | 6.2 |
| Pump | 14.0 |
| Intake | 1.9 |
| Fluid | 1.6 |

## 7. Final Health Score Result

Total degradation:

D_total = 37.1

Final Health Score:

S_health = 100 - 37.1

S_health = 62.9

Rounded dashboard value:

GUARDX Health Score = 62.9 / 100

## 8. Dashboard Interpretation

Status:
WARNING – Developing Degradation

Main Contributors:
1. Pump scaling and efficiency loss
2. Seal thrust-bearing wear
3. Motor imbalance
4. Moderate harmonic stress
5. Early gas interference

Recommended GUARDX Actions:
- Reduce operating frequency slightly toward BEP
- Monitor seal-section temperature trend
- Review harmonic filter and VSD switching behavior
- Schedule fluid treatment for scaling/asphaltene buildup
- Monitor gas-lock recurrence frequency

## 9. Operational Meaning

This is a realistic predictive-maintenance operating condition. The ESP system is still running and production continues normally, however GUARDX detects that several degradation mechanisms are simultaneously developing.

This operating region is where predictive AI provides the highest operational value because intervention can occur before catastrophic failure, unplanned shutdown, or expensive workover.